- Consider a single link with bandwidth B and propagation delay (i.e., latency) L. It takes 1ms for an entire 500 byte packet to arrive at other end of the link (that is, it takes 1ms from the time the first bit starts being transmitted until the last bit arrives at the other end of the link). It takes 2ms for an entire 1500 byte packet to arrive at the other end of the link.

- What is the bandwidth B of the link? (in Mbps)

- What is the propagation delay L of the link? (in ms) :

  - Total delay = 1ms

- Calculate the total time required to transfer a 1.5-MB file in the following cases, assuming a RTT of 80 ms, a packet size of 1 KB and an initial 2×RTT of “handshaking” before data is sent.

(a) The bandwidth is 10 Mbps, and data packets can be sent continuously.

total time = initial handshaking + network delay

initial handshaking = 2*RTT = 2*80 ms = 160 ms.

Delay = propagation delay + transmission delay (assuming processing & queuing

delays are not significant).

= 40 ms + (1.5-MB / 10 Mbps)

= 40 ms + (1.5*1,048,576 B / 10*1,000,000 bps)

= 40 ms + (1.5*1,048,576*8 b / 10*1,000,000 bps)

= 40 ms + (12,582,912 bits / 10,000,000 bps)

= 40 ms + 1.26 s

= 40 ms + 1260 ms

= 1300 ms

» total time = initial handshaking + network delay

= 160 ms + 1300 ms

= 1460 ms

= 1.46 s

(b) The bandwidth is 100 Mbps, but after we finish sending each data packet we must wait one RTT before sending the next.

First, count number of packets needed to send the file:

1.5-MB / 1 KB = 12,582,912 bits / 1024(8) bits

= 12,582,912 bits / 8192 bits

= 1536

Thus, we need (1536-1) RTTs to be added to the total delay time.

Second, we need to calculate the network delay in this case:

Delay = propagation delay + transmission delay

= 40 ms + (1.5-MB / 100 Mbps)

= 40 ms + (1.5*1,048,576 B / 100*1,000,000 bps)

= 40 ms + (1.5*1,048,576*8 b / 100*1,000,000 bps)

= 40 ms + (12,582,912 bits / 100,000,000 bps)

= 40 ms + 0.126 s

= 40 ms + 126 ms

= 166 ms

» total time = initial handshaking + network delay + 1535 (80 ms)

= 160 ms + 166 ms + 122800 ms

= 123,126 ms

= 123.26 s

- Assume you are able to download a movie on a website which stores the movie as seven separate video files. The total processing/service time within the network is 20ms and the one-way propagation delay is 120ms. Assume that transmission delay is negligible. Compute the time between when the browser on your machine requests the videos from the website and when they are delivered, if your browser uses:

(a) non-persistent HTTP?

(b) persistent HTTP?

- Host TestKingA needs to communications with the e-mail server shown above. What address will be placed on the destination address field of the frame when it leaves host TestKingA?

A. The MAC address of TestKingA
B. The MAC address of switch TestKing1
C. The MAC address of the E0 interface of the TestKing3 router. D. The MAC address of the E1 interface of the TestKing3 router. E. The MAC address of switch TestKing2
F. The MAC address of the email server

- Which of the following protocols operate at the 'Application layer' of the OSI model? (Select all valid answers)

A. TCP
B. Telnet
C. FTP
D. ARP
E. IP
F. None of the above

- At which layer of the OSI model is the optimal path to a network destination determined at?

A. Data Link B. Session
C. Physical
D. Presentation

E. Network

F. Transport

The TCP/IP network model differs from the more popular OSI model. Which of the layers below belong to the TCP/IP model? (Select all that apply)

A. Application layer

B. Session layer

C. Transport layer

D. Internet layer

E. Network layer

F. Data link layer

G. Physical layer